Karlheinz Stockhausen

MANTRA

for 2 pianos and electronics

Marco Stroppa, August 2015 (adapted from Serge Lemouton, 2012)

stroppa@marcostroppa.eu

ELECTRONIC RIDER

For this version, each pianist uses an iPad which controls the frequencies of the oscillators generated by pd.

NB: middle A = A4.

Needed software: Pd (on the Macintosh), TouchOSC (on both iPads).

Interfaces (controllers for the pianists)

Interface piano 1 (three pages):

- main rotary control, F#3 [54] – G#4 [68]. The yellow line below the rotary control allows for a visual fine tuning of the frequencies, but does not send any control values. The horizontal blue slider at the bottom shows the whole range of the piece, from appx. 3Hz to F#6 [90].

- red vertical slider for the high register (D4 [62]  F#6 [90], through A5 [81]).

- Kurzwellen, start/stop (left) and volume (brown vertical slider).

To move from one page to the other, tap the upper 0.5 cm on the right, centre or left.

Interface piano 2 (three pages):

- main rotary control, Bb2 (46) – C4 (60). The blue line below the rotary control allows for a visual fine tuning of the frequencies, but does not send any control values. The horizontal red slider at the bottom shows the whole range of the piece, from A2 [45] to A5 [81].

- red vertical slider for the high register (D4 [62]  A5 [81]).

- very low frequencies 5Hz [-8.51]  7Hz [-2.59]. NB. the start button is not effective!

To move from one page to the other, tap the upper 0.5 cm on the right, centre or left.

Setup of the Wi-Fi/OSC connection

between iPads and Mac (Pd)

A. INITIALIZATION

• reset the network settings on both iPads:

Settings / General / Reset (all the way below) / Reset Network Settings

This should clean any problematic settings and reduce synchronization issues.

• find the best Wi-Fi channel. To do this (tested on OSX 10.9.5):

- Turn Wi-Fi On

- Alt Click on the Wi-Fi icon and choose the last line: Open Wireless Diagnostics

- Give your password, but DO NOT Continue.

- Choose Window/Utilities (cmd 2)

- Select Wi-Fi Scan and type on Scan Now

- Use one of the proposed Best 2.4 GHz Channels when setting up the local network (see below)

B. ON THE MACINTOSH:

- In Network Preferences / Wi-Fi type on Advanced, then on TCP/IP. Configure the Network Manually as below:

- Create a local Network:

- Call the network Mantra, choose one of the best channels (Channel 6, here), use a 40-bit WEP security, and type a 5-character password (I use “ircam”):

- Make sure that you have the latest version of Pd installed (at this time: Pd-0.46-6_32). If not, download the Pd vanilla version (that is, the core of Pure Data, mainly written by Miller Puckette, focusing on audio signal and MIDI processing) on http://msp.ucsd.edu/software.html (verified on August 16, 2015).

- Open Audio MIDI Setup and verify that the audio driver and sampling rate are correct. The current version uses 96KHz, but there is also an older version running at 44.1KHz. The usage of a Fireface Audio Interface is strongly recommended.

- Go in mantra2015/mantra-pd and double click on the red patch (at this time: mantra2015i-96).

- On the main page, make sure that the local IP addresses are exactly as below:

- Open pd controllers-IPADS and make sure that the patch below the two loadbang’s looks exactly as below, as well as the udpreceive’s contain the numbers 8006 and 9997.

Leave Pd on while setting the iPads.

B. ON THE iPADs:

Initialization and installation

- If not already done, install TouchOSC (available from the Apple Store, current version: 1.9.4) on both iPads.

- From iTunes, copy on both iPads the following files (in mantra2015 / TouchOSC) into the Application by dragging them into the appropriate space: mantra_piano1b.touchosc, mantra_piano2b.touchosc.

Setting the communication.

NB: The following information applies to both iPads. When the settings differ, it is shown by a double picture.

- In Settings / Wi-Fi join the Network called Mantra (give the password you defined).

- Type on “i”, choose “Static” and give exactly the addresses below:

iPad1	iPad2

Leave Auto-Join ON, so that the Network will be automatically joined next time the iPad is turned on.

- Open TouchOSC and tap on the name below LAYOUT (here it is already selected):

- Select mantra_piano1b and mantra_piano2b from the list for the iPad n. 1 and 2.

iPad1	iPad2

- Go back to the previous page and check that the OSC address below CONNECTIONS is: 129.255.1.1 (that is, the address of the local network manually entered on the Macintosch). If it is not the case, a problem occurred or an error was made and the local network should be recreated.

- Type on OSC and set the in and outgoing ports as below:

iPad1	iPad2

- Go back to the main page (page 1 of the controller) on both iPads.

NB: If asked, reply “Trust” to the iPad’s question “Trust This Computer?”

C. FINALIZING THE COMMUNICATION

- Go back to Pd and click on both “disconnect, connect…” boxes to initialize the networks.

In the Pd window, the following messages should appear:

- Move the rotary slider on both iPads. The horizontal sliders below should also move.

iPad1	iPad2

- Check, in the main Pd patch, that the pitch and ring values also move:

If this is the case, the communication between the iPads and the Macintosh is correctly setup.

NB:

If the following message appears in the Pd window when moving the sliders:

this means that one or both iPads have connection problems. In this case, check that they are correctly turned on (no automatic sleep!), and click again on “disconnect, connect…” in the main window. If the message still appears, restart the patch and/or the iPads.